#### ASM Configuration

Prerequisites:-

Step 1:

- Set up Oracle Database Server on VMware Workstation using( OEL) Linux 7.6 OS Platform?

Step 2: Connect to OEL 7.6 server via putty and install the below packages

|  | yum -y install -y oracle-database-preinstall-19c |
| --- | --- |

- yum -y install oracleasm*

- yum -y install kmod-oracleasm*

Add below OS groups :

| 1 2 3 | groupadd -g 54327 asmdba groupadd -g 54328 asmoper groupadd -g 54329 asmadmin |
| --- | --- |

Add asmdba as secondary group to Oracle user

|  | usermod -a -G asmdba oracle |
| --- | --- |

Create Grid User

|  | useradd -u 54331 -g oinstall -G dba,asmdba,asmoper,asmadmin,racdba grid |
| --- | --- |

Change the password for Oracle and Grid user

|  | passwd oracle |
| --- | --- |

- passwd grid

Create the Directories for Oracle Database installation

| 1 2 3 | mkdir -p /u01/app/oracle mkdir -p /u01/app/oracle/product/19.3.0/db_home chown -R oracle:oinstall /u01/app/oracle |
| --- | --- |

Create the Directories for Oracle Grid installation

| 1 2 3 | mkdir -p /u01/app/grid/product/19.3.0/grid_home chown -R grid:oinstall /u01/app/grid chmod -R 775 /u01 |
| --- | --- |

Configure Oracle ASM

Let us configure Oracle ASM

|  | oracleasm configure -i |
| --- | --- |

Load / initiate Oracle ASM

- Fire  oracleasm init

Add 30 GB HDD to virtual machine which we will use to create 3 partitions of 10 GB each for our ASM disks. Open Virtual machine

Stop the server go to >> Settings >> Storage >> Controller: SATA >> Create new disk

create 3 partitions of 10 GB each for our ASM disks

login as root user to format the  disk

|  | fdisk -l |
| --- | --- |

In the above output the /dev/sdb is not yet formatted. Let’s create 3 partitions on sdb